Configuring IP Phones on a Local Network

Topology

PC Port

Fast Ethernet Port

Fast Ethernet Port

PC Port

PH 2

PH 1

R1

S1

Fe0/0

Fe0/48

Fe0/1

Switch Port

Fe0/2

Switch Port

PC-B

PC-A

Background

Businesses use IP phones to allow voice calling between departments via their internal computer network. This lab will walk you through setting up a local network in which both IP phones and computers can communicate with each other.

Equipment List

- 1 Cisco 2811 Router

- 1 Cisco 2960 or 3560 series POE-48 Switch

- 2 Cisco 7945 IP Phones

- 2 PCs with RJ-45 ports

- 5 Lengths of straight through RJ-45 cable

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| S1 | VLAN 10 (data) | 192.168.10.0 | 255.255.255.0 | N/A |
| S1 | VLAN 20 (voice) | 192.168.20.0 | 255.255.255.0 | N/A |
| S1 | VLAN 30 (mgt) | N/A | N/A | N/A |
| S1 | VLAN 40 (misc.) | N/A | N/A | N/A |
| S1 | VLAN 50 (trunk) | N/A | N/A | N/A |
| R1 | Fe0/0.10 | 192.168.10.1 | 255.255.255.0 | N/A |
| R1 | Fe0/0.20 | 192.168.20.1 | 255.255.255.0 | N/A |
| R1 | Fe0/0.50 | N/A | N/A | N/A |
| R1 | DATA10 dhcp pool | 192.168.10.0 | 255.255.255.0 | 192.168.10.1 |
| R1 | VOICE 20 dhcp pool | 192.168.20.0 | 255.255.255.0 | 192.168.20.1 |
| PC-A | NIC | 192.168.10.3 | 255.255.255.0 | 192.168.10.1 |
| PC-B | NIC | 192.168.10.4 | 255.255.255.0 | 192.168.10.1 |

Objectives

Part 1: Interface VLANs on the switch

- Console into the Switch

- Interface VLAN10 for data

- Interface VLAN20 for voice

- Interface VLAN 30 for management

- Interface VLAN 40 for the unused ports

- Interface VLAN 50 as a trunk.

Part 2: Interface the ports on the switch

- Interface port fa0/48 as a trunk

- Interface port(s) fa0/1-2

- Interface port(s) fa0/3-47

- Interface port(s) gig0/1-4 & save the configuration

- Connect the switch to the router

Part 3: Interface VLAN routing

- Console into the Router

- Configure sub interface routing for VLAN10 on port Fa0/0.10

- Configure sub interface routing for VLAN20 on port Fa0/0.20

- Configure sub interface routing for VLAN50 (trunk) on port Fa0/0.50

- Turn on port Fa0/0

Part 4: Configure DHCP for VLANs

- Excluding addresses for VLAN10 & VLAN20

- Configuring the IP addresses & subnet masks for VLAN10 and VLAN20

Part 5: Configure telephony services

- Configure the directory for each ephone

- Configure a number for each ephone

Part 6: Configure ephones 1 & 2

- Configure ephone 1’s mac address, type, and button number

- Configure ephone 2’s mac address, type, and button number

- Save the configuration & connect the IP phones to the switch

- Verify phone connectivity

Part 7: Configure PC Hosts

- Enable DHCP on PCs

- Connect the PCs to the Phones

- Verify PC connectivity

Instructional

Part 1: Interface VLANs on the Switch.

Step 1: Console into the Switch.

- Plug the RJ-45 terminated end, of the Console Cable, into the Console Port on the 2960/3560 Switch.

- Open Terra Term on the PC.

- Select Serial, open the drop-down menu, select the Switch COM Port, and select OK.

Step 2: Interface VLAN 10 for data.

- Enter Privileged EXEC mode using the Enable command.

Switch>enable

- Enter Terminal Configuration mode using Configure Terminal command.

Switch#configure terminal

- Configure the name for VLAN 10.

Switch(config)#vlan 10

Switch(config-vlan)#name DATA

Step 3: Interface VLAN 20 for voice.

- Configure the name for VLAN 20.

Switch(config-vlan)#vlan 20

Switch(config-vlan)#name VOICE

Step 4: Interface VLAN 30 for management.

- Configure the name for VLAN 30.

Switch(config-vlan)#vlan 30

Switch(config-vlan)#name MGT

Step 5: Interface VLAN 40 for the unused ports.

- Configure the name for VLAN 40.

Switch(config-vlan)#vlan 40

Switch(config-vlan)#name MISC

Step 6: Interface VLAN 50 as a trunk.

- Configure the name for VLAN 50.

Switch(config-vlan)#vlan 50

Switch(config-vlan)#name NATIVE

- Exit the VLAN configuration mode using the Exit command.

Switch(config-vlan)#exit

- Show the Vlan configuration using the Do Show Vlan command.

Switch(config)#do show vlan

- You should see 5 VLANs named: DATA, VOICE, MGT, MISC, and NATIVE.

Part 2: Interface the Ports on the Switch.

Step 1: Interface Port Fa0/48 as a Trunk.

- Enter the interface for port Fa0/48 using the Interface command.

Switch(config)#interface fa0/48

- Ensure that the trunk encapsulation is dot1Q. (*NOTE* if this results in an error, this step is unnecessary because dot1Q encapsulation is the only available option)

Switch(config-if)#switchport trunk encapsulation dot1Q

- Change the Switchport Mode to Trunk so that one port can carry traffic for all the VLANs.

Switch(config-if)#switchport mode trunk

- Change the Native Trunk to VLAN 50.

Switch(config-if)#switchport trunk native vlan 50

- Exit the interface mode for port Fa0/48 using the Exit command.

Switch(config-if)#exit

Step 2: Interface ports Fa0/1-2

- Enter the interface(s) for port(s) Fa0/1 & Fa0/2 using the Interface Range command.

Switch(config)#interface range fa0/1-2

- Turn on power over ethernet (PoE) using the Power Inline Auto command.

Switch(config-if-range)#power inline auto

- Disable dynamic trunking on port(s) Fa0/1 & Fa0/2 using the Switchport Mode Access command.

Switch(config-if-range)#switchport mode access

- Change the VLAN that the Access Port carries traffic to VLAN 10.

Switch(config-if-range)#switchport access vlan 10

- Set Vlan 20 as the voice VLAN using the Switchport Voice VLAN command.

Switch(config-if-range)#switchport voice vlan 20

- Exit the interface range mode for port(s) Fa0/1 through Fa0/2 using the Exit command.

Switch(config-if-range)#exit

Step 3: Interface port(s) Fa0/3-47.

- Enter the interface(s) for port(s) Fa0/3 through Fa0/47 using the Interface Range command.

Switch(config)#interface range fa0/3-47

- Set the Access to Vlan 40.

Switch(config-if-range)#switchport access vlan 40

- Shut down the unused ports using the Shutdown command.

Switch(config-if-range)#shutdown

- Exit the interface range mode for port(s) Fa0/3 - Fa0/47 using the Exit command.

Switch(config-if-range)#exit

Step 4: Interface port(s) Gig0/1-4 & save the configuration.

- Enter the interface(s) for port(s) Gig0/1 through Gig0/4 using the Interface Range command.

Switch(config)#interface range gig0/1-4

- Set the Access to Vlan 40.

Switch(config-if-range)#switchport access vlan 40

- Secure the unused ports by shutting them down using the Shutdown command.

Switch(config-if-range)#shutdown

- Return to Privileged EXEC mode using the End command.

Switch(config-if-range)#end

- Change the startup configuration to the current running configuration using the Copy Run Start command. Press Enter at the prompt.

Switch#copy run start

- Show the running configuration using the Show Run command.

Switch#show run

- FastEthernet0/1 & 0/2 should be set to Switchport Access VLAN 10, Switchport Mode Access, and set to Switchport Voice VLAN 20. FastEthernet 0/3 - 47 & GigabitEthernet 0/1 – 4 should be set to Switchport Access VLAN 40 and Shutdown. You should see FastEthernet 0/48 set to switchport trunk native VLAN 50 and set to switchport mode Trunk. **if you had to set the encapsulation method then you will also see switchport mode Trunk encapsulation dot1Q**

Step 5: Connect the Switch to the Router

- Plug one RJ-45 end of a Straight Through Copper Cable into port Fe0/48 on the 2960/3560 Switch.

- Plug the other RJ-45 end into port Fe0/0 on the 1811 Router.

Part 3: Interface VLAN Routing

Step 1: Console into the Router.

- Plug the RJ-45 terminated end, of the Console Cable, into the Console Port on the 2811 Router.

- Open Terra Term on the PC.

- Select Serial, open the drop-down menu, select the Router COM Port, and select OK.

Step 2: Configure Sub Interface Routing for VLAN10 on Port Fa0/0.10.

- Enter Privileged EXEC mode using the Enable command.

Router>enable

- Enter Terminal Configuration mode using Configure Terminal command.

Router#configure terminal

- Enter the Sub Interface for port Fa0/0.10 using the Interface command.

Router(config)#int fa0/0.10

- Set Encapsulation type to dot1Q.

Router(config-subif)#encapsulation dot1Q 10

- Assign an IP address and Subnet Mask to the Sub Interface.

Router(config-subif)#ip address 192.168.10.1 255.255.255.0

- Exit the Sub Interface with the Exit command.

Router(config-subif)#exit

Step 3: Configure Sub Interface Routing for VLAN20 on port Fa0/0.20.

- Enter the Sub Interface for port Fa0/0.20 using the Interface command.

Router(config)#int fa0/0.20

- Set Encapsulation type to dot1Q.

Router(config-subif)#encapsulation dot1Q 20

- Assign an IP address and Subnet Mask to the Sub Interface.

Router(config-subif)#ip address 192.168.20.1 255.255.255.0

- Exit the Sub Interface with the Exit command.

Router(config-subif)#exit

Step 4: Configure Sub Interface Routing for VLAN50 (trunk) on port Fa0/0.50.

- Enter the Sub Interface for port Fa0/0.50 using the Interface command.

Router(config)#int fa0/0.50

- Set Encapsulation type to dot1Q.

Router(config-subif)#encapsulation dot1Q 50 native

- Exit the Sub Interface with the Exit command.

Router(config-subif)#exit

Step 5: Turn on port Fa0/0.

- Enter the interface for port Fa0/0 using the Interface command.

Router(config)#interface fa0/0

- Turn on the interface with the No Shutdown command.

Router(config-if)#no shutdown

- Exit the interface mode for port Fa0/0 using the Exit command.

Router(config-if)#exit

- Show the running configuration using the Do Show Run command.

Router(config)#do show run

- Interface FastEthernet0/0 should no longer be Shutdown. Three sub interfaces should show up: FastEthernet0/0.10, FastEthernet0/0.20, and FastEthernet0/0.50. They should be set to encapsulation dot1Q 10, 20, & 50. Each sub interface should be assigned an IP address and Subnet Mask.

Part 4: Configure DHCP for the VLANs.

Step 1: Excluding addresses for VLAN10 & VLAN20.

- Exclude the first 2 addresses of the 192.168.10.0 Network using the Ip dhcp excluded-address command.

Router(config)#ip dhcp excluded-address 192.168.10.1 192.168.10.2

- Exclude the first 2 addresses of the 192.168.20.0 Network using the Ip dhcp excluded-address command.

Router(config)#ip dhcp excluded-address 192.168.20.1 192.168.20.2

Step 2: Configuring the IP addresses & subnet masks for VLAN10 and VLAN20.

- Enter the DHCP configuration for DATA10 using the Ip dhcp pool command.

Router(config)#ip dhcp pool DATA10

- Give the pool the 192.168.10.0 network address and the 255.255.255.0 subnet mask using the Network command.

Router(dhcp-config)#network 192.168.10.0 255.255.255.0

- Assign the default router for this network using the default-router command.

Router(dhcp-config)#default-router 192.168.10.1

- Enter the DHCP configuration for DATA20 using the Ip dhcp pool command.

Router(dhcp-config)#ip dhcp pool VOICE20

- Give the pool the 192.168.20.0 network address and the 255.255.255.0 subnet mask using the Network command.

Router(dhcp-config)#network 192.168.20.0 255.255.255.0

- Assign the default router for this network using the default-router command.

Router(dhcp-config)#default-router 192.168.20.1

- Assign option 150 to allow VoIP phones to obtain the configuration server address.

Router(dhcp-config)#option 150 ip 192.168.20.1

- Exit DHCP configuration mode using the Exit command.

Router(config-if)#exit

- Show the running configuration using the Do Show Run command.

Router(config)#do show run

- There should be two DHCP pools. The first should be named DATA10 with the Network address: 192.168.10.0, Subnet Mask: 255.255.255.0, and Default Router: 192.168.10.1. The second should be named VOICE20 with the network address of 192.168.20.0, Subnet Mask: 255.255.255.0, and Default Router: 192.168.20.1. The first two addresses of each network should be excluded.

Part 5: Configure Telephony Services.

Step 1: Configure the Directory for each Phone.

- Enter Telephony Configuration mode using the Telephony-Services command.

Router(config)#telephony-service

- Assign a telephony source Ip address and Port #.

Router(config-telephony)#ip source-address 192.168.20.1 port 2000

- Assign a maximum number of directory entries using the max-dn command.

Router(config-telephony)#max-dn 2

- Assign a maximum number of ephones using the max-ephones command.

Router(config-telephony)#max-ephones 2

Step 2: Configure a number for each ephone.

- Assign a number to the first ephone in the directory.

Router(config-telephony)#ephone-dn 1

Router(config-ephone-dn)#number 1010

- Assign a number to the second ephone in the directory.

Router(config-ephone-dn)#ephone-dn 2

Router(config-ephone-dn)#number 1020

- Exit telephony services configuration mode using the Exit command.

Router(config-ephone-dn)#exit

Part 6: Configure Ephones 1 & 2

Step 1: Configure ephone 1’s MAC address, type, and button number

- Enter ephone 1’s configuration mode by entering ephone 1 at the terminal configuration mode.

Router(config)#ephone 1

- Enter the phone’s MAC address located on the Rear of the phone.

Router(config-ephone)#mac-address xxxx.xxxx.xxxx

- Enter the phone type 7945 and button number 1:1.

Router(config-ephone)#type 7945

Router(config-ephone)#button 1:1

Step 2: Configure ephone 2’s MAC address, type, and button number.

- Enter ephone 2’s configuration mode by entering ephone 2.

Router(config-ephone)#ephone 2

- Enter the phone’s MAC address located in the same spot on the Rear of the other phone.

Router(config-ephone)#mac-address xxxx.xxxx.xxxx

- Enter the phone type 7945 and button number 1:2.

Router(config-ephone)#type 7945

Router(config-ephone)#button 1:2

- Return to user EXEC mode by entering the End command

Router(config-ephone)#end

Step 3: Save the Configuration & Connect the IP Phones to the Switch.

- Change the startup configuration to the current running configuration using the Copy Run Start command.

Router#copy run start

- Show the running configuration using the Show Run command.

Router#show run

- Under Telephony-Service, the IP Source-Address should be 192.168.20.1 on Port 2000. Ephone 1 should be in the directory as number 1010 and Ephone 2 should be in the directory as number 1020. Ephones 1 & 2 should have their MAC Addresses listed and be listed as Type 7945. Ephone 1 should be button 1:1 and ephone 2 should be button 1:2.

- Plug a Straight Through Copper Cable into the RJ-45 Switch port on each IP phone.

- Plug the other end of each Straight Through Copper Cable into either port fa0/1 or port fa0/2 on the Switch.

Step 4: Verify Phone Connectivity

- Pick up a phone and dial the number not assigned to that phone. (1010 or 1020)

- The other phone should ring.

Part 7: Configure PC Hosts

Step 1: Enable DHCP on the PCs

- Click the windows icon on the taskbar, select settings

- Select Network & Internet

- Select Ethernet Properties, Select Edit IP settings, Select DHCP, and select Save.

Step 2: Connect the PCs to the Phone

- Plug the RJ-45 terminated end of Straight Through Copper Cable into the PC Port on each IP phone.

- Plug the other RJ-45 end into the Ethernet port on each PC.

Step 3: Verify PC connectivity using the ping utility.

- In the Windows Search Bar type Command Prompt and open the Best Match.

- Type “ping 192.168.10.X” and press enter (X is the IP address of the other PC)

- View successful pings