Configuring a Raspberry Pi to be a “Honeypot” for Intrusion Detection Using Opencanary.

Background

A great way to detect threats to you or your company’s network is the use of a “honeypot.” A “Honeypot” is a term for a vulnerable machine on your network, usually masquerading as a server with open ports. This is intended to entice hackers who may have already breached your network to attack it and thus notifying the person(s) in charge of securing the network.

Prerequisites

- Raspberry Pi 3 or 4

- PC

- A Micro SD card

- USB Keyboard & HDMI monitor

- A Gmail Account

Index

Part 1: Install Rasbian on an SD card.

- Download Rasbian from the official website.

- Download Balena Etcher.

- Flash the Raspbian ISO to the SD card with Balena Etcher.

- Insert the SD Card into the Raspberry Pi and Power On the device.

Part 2: Create Login credentials, Configure, and Update the OS.

- Plug a USB keyboard into the RaspberryPi. Follow the prompts to choose an Account Name and Password.

- Create a Bash Script to change the MAC address of the Raspberry Pi.

- Enter the root account with the command sudo bash. Now run this script using the command source chadd.sh. Type the command exit to leave the root file system. Type the command cd ~ to return to the home directory.

- Update the repositories of the OS by using the command sudo apt update && sudo apt upgrade -y.

Part 3: Connect to the RaspberryPi remotely via SSH, “Secure Shell.”

- Find out the device’s IP address using the command ifconfig.

- Connect remotely using Linux.

- Connect remotely using Windows.

Part 4: Install Python tools & Samba.

- Use the apt-get command to install python3-dev, pip, virtualenv, venv, scapy, libssl-dev, and libpcap-dev.

- Use the apt command to install Samba.

Part 5: Create and name the Python virtual environment & activate the environment.

- Use the python3 venv command to create the virtual environment using the –m flag to name it.

- Type . canary/bin/activate to activate the virtual environment.

Part 6: Install OpenCanary, Scapy, Pcapy, and Markupsafe in the virtual environment.

- Use the pip3 command to install opencanary.

- Use the pip3 command to install scapy and pcapy.

- Use the pip3 command to install markupsafe with the -Iv flag to select version 2.0.1.

Part 7: Setup a Gmail app password.

- Select Accounts. Select Manage Your Google Account.

- Select Security on the Top Bar. Scroll down and select 2-Step Verification under the sub menu called Signing In To Google.

- Click Get Started. Enter your Password and click Next. Select a device to get prompts and click Continue.

- Set up a Backup Option & click Send. Enter the Verification Code you recieved. Once verified click the return Arrow.

- Scroll down and select App Passwords. Open the Select App drop down menu & Select Other.

- Type OpenCanary in the app name field and click Generate. Copy the generated App Password to a Text File and return to the Terminal.

Part 8: Copy & Modify the Configuration file. Add SMTP alerts.

- Use the opencanaryd command with the command line argument copyconfig to make a copy of the configuration file.

- Open and edit the configuration file using vi or nano.

- Change the “device.node_id”: to “SynologyFS”. Ensure “ftp.enabled”: & “http.enabled”: are both True.

- Ensure “ssh.enabled”:  says  True.

- To enable Email allerts add SMTP under “handlers”.

- Press Ctrl+x, Ctrl-y, and press Enter to save the changes.

Part 9: Run OpenCanary and test event Logging.

- SSH services in the OS must be disabled before opencanary can run with it’s own SSH services. To Disable this use the command sudo raspi-config. Select Interface options, SSH, and No.

- Use the opencanaryd command with the command line argument start to run opencanary.

- Attempt an ssh, ftp and https connection with the device using any credentials.

- Check email or the logging file /var/tmp/opencanary.log.

Instructional

Part 1: Install Rasbian on an SD card.

- Download Rasbian from the official website: https://downloads.raspberrypi.org/raspios_lite_armhf/images/raspios_lite_armhf-2022-04-07/2022-04-04-raspios-bullseye-armhf-lite.img.xz.

- Download Balena Etcher from: https://www.balena.io/etcher/.

- Flash the Raspbian ISO to the SD card with Balena Etcher.

- Insert the SD Card into the Raspberry Pi and Power On the device.

Part 2: Create Login credentials, Configure, and Update the OS.

  - Plug a USB keyboard into the RaspberryPi and connect to an HDMI monitor. Follow the prompts choose UK English, a Username and a Password.

  - Create a Bash Script to change the MAC address of the Raspberry Pi.

    - Make a new directory named address using the command mkdir address. Change your working directory to this new directory using the command cd address. Create a new text file in this directory by using the nano command.

    - Enter the text shown below. Press ctrl+x, ctrl+y, type chadd.sh, and press Enter.

  - Enter the root account with the command sudo bash. Now run the file by using the command source chadd.sh. Type the command exit to leave the root file system. Type the command cd ~ to return to the home directory.

  - Update the repositories of the OS by using the command sudo apt update && sudo apt upgrade -y.

Part 3: Connect to the RaspberryPi remotely via SSH, “Secure Shell.”

  - SSH services in the OS must be Enabled before opencanary can run with it’s own SSH services. To Enable this use the command sudo raspi-config. Select Interface options, SSH, and Yes.

- Find out the device’s IP address using the command ifconfig.

- Connect remotely using Linux.

  - Open the terminal of a computer running linux. Use the ssh command in the format username@ipaddress.

- Connect remotely using Windows.

  - Install and Load TerraTerm on a PC. Put the IP address you found into the Host section & click Okay.

  - Click Continue when prompted with a security warning. Type in the Username and the Password you set up. Click Okay.

Part 4: Install Python tools & Samba.

  - Use the apt-get command to install python3-dev, pip, virtualenv, venv, scapy, libssl-dev, and libpcap-dev.

  - Use the apt command to install Samba.

Part 5: Create and name the Python virtual environment & activate the environment.

  - Use the python3 venv command to create the virtual environment using the –m flag to name it canary.

  - Type . canary/bin/activate to activate the virtual environment.

Part 6: Install OpenCanary, Scapy, Pcapy, and Markupsafe in the virtual environment.

  - Use the pip3 command to install opencanary.

  - Use the pip3 command to install scapy and pcapy.

  - Use the pip3 command to install markupsafe with the -Iv flag to select version 2.0.1.

Part 7: Setup a Gmail app password.

  - Select Accounts. Select Manage Your Google Account.

  - Select Security on the Top Bar. Scroll down and select 2-Step Verification under the sub menu called Signing In To Google.

  - Click Get Started. Enter your Password and click Next. Select a device to get prompts and click Continue.

  - Set up a Backup Option & click Send. Enter the Verification Code you recieved. Once verified click the return Arrow.

  - Scroll down and select App Passwords. Open the Select App drop down menu & Select Other.

  - Type OpenCanary in the app name field and click Generate. Copy the generated App Password to a Text File and return to the Terminal.

Part 8: Copy & Modify the Configuration file. Add SMTP alerts.

  - Use the opencanaryd command with the command line argument copyconfig to make a copy of the configuration file.

  - Open and edit the configuration file using vi or nano.

  - Change the “device.node_id”: to “SynologyFS”. Ensure “ftp.enabled”: & “http.enabled”: are both True.

  - Scroll down and ensure “ssh.enabled”: also says True.

  - To enable Email Alerts add this text under “handlers”. https://opencanary.readthedocs.io/en/latest/alerts/email.html

"handlers": {

"SMTP": {

"class": "logging.handlers.SMTPHandler",

"mailhost": ["smtp.gmail.com", 25],

"fromaddr": "noreply@gmail.com",

"toaddrs" : ["youraddress@gmail.com"],

"subject" : "OpenCanary Alert",

“credentials” : [“youraddress@gmail.com”, “apppassword”]

“secure” : []

},

  - Press Ctrl+x, Ctrl-y, and press Enter to save the changes.

Part 9: Run OpenCanary and test event Logging.

  - SSH services in the OS must be disabled before opencanary can run with it’s own SSH services. To Disable this use the command sudo raspi-config. Select Interface options, SSH, and No.

  - Disconnect from SSH and Physically access the Pi.

  - Use the opencanaryd command with the command line argument start to run opencanary.

  - Attempt an ssh, ftp and https connection with the device using any credentials.

  - Check email or the logging file /var/tmp/opencanary.log.